Project Title: Medicine Reminder System Using LPC2148

Overview: This embedded project is a Real-Time Clock (RTC) based Medicine Reminder System built on the LPC2148 microcontroller. It keeps track of time using RTC and alerts the user with a buzzer when it's time to take their medicine. The system also includes a menu interface via keypad for setting time and medicine reminders, and a 16x2 LCD for user display.

Features:

- Display current time, date, and day on an LCD.

- Set and edit RTC time and medicine reminder time.

- Buzzer alert at the set medicine time.

- User interaction via push buttons and a keypad.

Hardware Requirements:

- LPC2148 ARM7 Development Board

- 16x2 LCD

- Matrix Keypad (4x4 or 4x3)

- Buzzer

- Push Buttons (Menu and Stop Alarm)

- Power supply module

- Supporting components (resistors, wires, etc.

Pin Configuration:

- BUZZER: P0.5

- Menu Button (sw1): P0.19

- Stop Alarm Button (sw2): P0.6

Included Files:

- Main_Project.c – main program source code

- delay.h – delay routines

- header.h – header for custom functions like LCD, RTC, keypad, edit functions, etc.

Code Flow Summary:

- Initialization:

- Configure Buzzer output pin.

- Initialize RTC, LCD, and Keypad.

- Set default time and date.

- Main Loop:

- Continuously fetch and display current time, date, and day on LCD.

- Detects Menu Button press (sw1) to enter setting mode:

- Option 1: RTC time editing.

- Option 2: Set medicine reminder time.

- Option 3: Exit menu.

- If current time equals medicine reminder time:

- Activate buzzer.

- Show "!MEDICINE TIME!" message.

- Wait for Stop Alarm Button (sw2) press to deactivate buzzer.

Functions Used:

- RTC_Init() – Initialize RTC

- RTCSetTime(h,m,s) – Set time

- RTCSetDate(d,m,y) – Set date

- RTCSetDay(day) – Set weekday

- RTCGetTime() / RTCGetDate() / RTCGetDay() – Fetch current time info

- DisplayRTCTime() / DisplayRTCDate() / DisplayRTCDay() – Display functions

- RTC_edit() – Edit RTC time via keypad

- medicine_edit() – Set alarm time via keypad

- delay_s(seconds) – Delay function

- keyscan() – Read keypad input

Usage Instructions:

- Power on the system.

- The current time/date/day is displayed on the LCD.

- Press the Menu Button (sw1) to enter menu:

- Press 1 for RTC edit.

- Press 2 to set medicine reminder.

- Press 3 to exit.

- At the set medicine time, the buzzer sounds.

- Press Stop Button (sw2) to stop the buzzer.